Table 1. Danger Index Factor Definitions and Data Sources

| Factor | Data source | Direction of danger | Spatial method |
| --- | --- | --- | --- |
| Ambient summer temperature | PRISM Climate Group (July max. temp., 2014–2023 avg.) | Hotter = more dangerous | 4 km raster, averaged to grid cell |
| Distance to major city | Straight-line to Phoenix or Tucson | Farther = more dangerous | Euclidean, degree-space |
| Distance to major road | Straight-line to nearest Interstate/ US/State highway (TIGER/Line 2021) | Farther = more dangerous | Euclidean, degree-space |
| Distance to water source | Straight-line to nearest Humane Borders water station | Farther = more dangerous | Euclidean, degree-space |
| Slope | USGS 3DEP elevation (slope computed from raw elevation) | Steeper = more dangerous | ~10× oversampled, block-averaged |
| Vegetation density (NDVI) | USGS NAIP 4-band imagery, (NIR−Red)/(NIR+Red) | Denser = more dangerousa | ~1 m native, block-averaged |

Note. PRISM = PRISM Climate Group, Oregon State University. TIGER/Line = U.S. Census Bureau topological road/boundary files. USGS 3DEP = U.S. Geological Survey 3D Elevation Program. NAIP = National Agriculture Imagery Program. NDVI = Normalized Difference Vegetation Index.

Note. ᵃDenser vegetation is scored as more dangerous, not as protective shade, following Boyce, Chambers, and Launius (2019): dense vegetation slows travel, disorients, and increases physical exertion.

Table 2. Descriptive Statistics for Danger Index Factors (N = 7,536 Grid Cells)

| Factor | Min | M | Max | SD | Unit |
| --- | --- | --- | --- | --- | --- |
| Ambient summer temperature | 20.6 | 37.1 | 42.9 | 4.2 | °C |
| Distance to major city | 1.2 | 79.6 | 183.0 | 39.0 | mi |
| Distance to major road | 0.0 | 7.4 | 42.5 | 6.5 | mi |
| Distance to water source | 0.3 | 75.4 | 194.9 | 43.2 | mi |
| Slope | 0.01 | 3.10 | 17.79 | 3.03 | deg |
| Vegetation density (NDVI) | −0.046 | 0.063 | 0.300 | 0.051 | unitless |

Note. M = mean; SD = standard deviation (n−1). Distance figures are Euclidean (straight-line) distances converted from degree-space to approximate miles using the study area's mid-latitude; not geodesic or road-network distances. All values are the raw, pre-Z-score inputs to the composite danger index (the sum of six standardized [Z-score] factors).